Kyle

Aero 4630

Homework 1

- For each statement below about Newton’s second law, mark “T” if the statement is true and “F” if the statement is false. A statement is false if it is not always true.

- Forces on an object are independent of its acceleration. (F)

- An object in motion will stop unless acted upon by a force. (F)

- The net force on an object is proportional to its mass times its acceleration. (T)

- The net force on an object equals zero at all times. (F)

- A linear damper with a damping constant of b=10Ns/mm is subject to a constant force F=1N at t=0. Which of the following best describes the motion of the damper? Why?

B. The damper keeps extending at the speed of 0.1 m/s forever

∑F=ma

Fext-bv=ma

1N-10Nsmmv=0

v=0.1ms

- Two linear springs are connected in parallel as shown. You can assume that when F=0, both springs are at their relaxed lengths and the extensions x1=x2=xrb=0.

- Write down the constitutive laws for both springs, the force balance equation for the connection on the right, and the geometric continuity equation for this system.

Fk1=k1x1

Fk2=k2x2

∑F=0→-Fk1-Fk2+F=0

x1=x2=xrb

- If the red box were replaced with a single spring, find the spring constant of that spring, krb.

F=Fk1+Fk2

F=k1x1+k2x2

F=k1xrb+k2xrb

F=k1+k2xrb

k1+k2xrb=krbxrb

krb=k1+k2

- Five springs are arranged between two endpoints, as shown. The springs are individually well desribed by Hooke’s law of the form f1=k1x1

- Write down the force balance equation for the connection labeled B.

f3+f2-f1=0

- True false, which of the followinga re correct geometric continuity equations if the points a and d are fixed?

- v2+v3+v4=0 F

- v2+v4=v5 F

- v1+v2+v5=0 T

- v3=v4F

- v1+v3+v4+v5=0T

- Write a correct geometric continuity equation involving v5 if the absolute velocities of points A and D are nonzero and equal to vA and vD, respectively.

v1+v2+v5=vd-va

- The springs and dampers in the system below are governed by the linear constitutive laws f=kx and f=bv, k>0,b>0, with possibly different k and b for the different elements. The vertical plates are connections and translate only. The walls are stationary. The elements may be moving. Forces, displacements, and velocities are defined according to our usual positive conventions. Fext is an externally applied force.

- Draw the free body diagrams for each connection point, indicating the forces acting on it and their direction.

- For each statement below, mark “T” if the statement is true and “F” if the statement is false.

- f1=f2 F

- f3=f5F

- f4=f5T

- f3+f5=f6T

- f2=f6T

- f3+f4=f2T

- Fext+f1-f2=0T

- v1=v2f

- v3-v4-v5=0T

- v3+v4=v6F

- v1+v2+v3+v4+v5+v6=0F

- If v4>0, then x5>0T

- If v3>0, then f3>0F

- Consider a perfectly linear spring (s) with k=10N/m and a perfectly linear damper (d) with b=10Ns/m in the arrangement below.

- If the applied force is a constant, F = 100N, find vd,vs and xs. Note that vd and vs are the relative velocities between the ends of the damper and the spring respectively