Kyle Kasmiersky

Prof. Luo

Aero 4630

Homework 2

- Find the general solution of the following homogenous ODE for a spring-damper system where Fext=0 and no initial conditions are prescribed

xt=-3x(t)

dxdt=-3x

dx-3x=dt

-13ln-3x=t+C1

-3x=e-3t+-3C1

x=-C23e-3t

- Consider the same system with a constant external force Fext=1, Obtain the general solution and verify it by plugging it back to the ODE. Provide a physical explanation of the solution.

xt=-3xt+1

dxdt=-3x+1

dx-3x+1=dt

-13ln-3x+1=t+C1

-3x+1=e-3t+3C1

x=e-3t+3C1-1-3

x=C2e-3t-1-3

x=C2e-3t

Check : C2e-3t=-3C2e-3t-1-3+1=C2e-3t-1+1=C2e-3t, valid

At infinity, the spring comes to rest at x=1/3, which mean’s its k value is 3. It should decrease slower than a’s ODE at a given starting position.

- Now, consider the same system, but the external force Fext=sint, which varies in time. Obtain its general solution and verify it by plugging it back into the ODE. Provide a physical explanation of the solution

xt=-3xt+sin⁡(t)

xt+3xt=sin⁡(t)

Undetermined coefficients

xp=Asint+Bcost

Acost-Bsint+3Asint+3Bcost=sint

3A-B=1

A+3B=0

-9B-B=1, B= -110

A=310

xp=310sint-110cost

x=Ce-3t+310sint-110cost

Check

xt+3xt=sin⁡(t)

-3Ce-3t+310cost+110sint+3Ce-3t+910sint-310cost=sin⁡(t)

310cost+110sint+910sint-310cost=sin⁡(t)

110sint+910sint=sin⁡(t)

1010sint=sint

The equation should start out larger, then decrease in magnitude as t approaches infinity because of Ce-3t, where it will settle on whatever sinusoid is created by the 310sint-110cost terms.

- For the same system as in part c, were Fext = sin(t) consider the initial condition of x(0) = 0.3. obtain the solution to the ode and verify it does satisfy the initial condition.

x=Ce-3t+310sint-110cost

0.3=C+0-110

C=0.4

x=410e-3t+310sint-110cost

0.3=410+0-110=310=0.3

- Find the solution to the following ode without directly solving it. Use the principle of superposition and the solutions you already have for parts b and c. verify your solution by plugging it back into the ODE and checking the boundary condition.

xt=-3xt+2sint+6, x0=0

x=C2e-3t-1-3

x=Ce-3t+310sint-110cost

Superposition answer:

x=Ce-3t+2310sint-110cost+613

x=Ce-3t+35sint-15cost+2

0=C-15+2

C=-95

x=-95e-3t+35sint-15cost+2

Check

275e-3t+35cost+15sint=275e-3t-95sint+35cost-6+2sint+6

15sint=-95sint+2sint=15sint

0=-95+0-15+2=0

- Use Euler integration Δt=0.3 in Matlab to approximate the solution to problem 1e from 0<t<10.

- Use Euler integration Δt=0.1 in Matlab to approximate the solution to problem 1e from 0<t<10.

- Plot the analytical solution from 2(e), Euler integration approximation from part 2(a) and 2(b) in a single plot in Matlab. Compare these three solutions and discuss the results you have.

Reducing the timestep made the Euler integration approach the analytical solution to the problem, although there is still visible error on the 0 to 10 time scale. I am glad to see they fit pretty close though as it gives me confidence in my analytical solution.

- Use Euler integration to approximate the solution to the ODE. Compare the results and discuss your observations.

The most glaring observation is that the largest timestep, instead of converging like the rest of the integrations actually diverged because of its error, besides that, 0.15 and 0.02 both converged to 0 while it appears that 0.2 might oscillate around 0 as t approaches infinity.

- Consider a vector v in the xy-plane with coordinates (x, y). We wish to rotate this vector counter-clockwise by an angle θ about the origin to obtain a new vector v′ with coordinates (x′, y′)

- Let the standard basis vectors be e1 and e2. After rotation by θ, these become e1’ and e2’. Using trigonometry, find the coordinates of e1’. Similary, find the coordinates of e2’.

e1'=cosθsinθ;e2'=-sinθcosθ

For θ=0

e1=10,e2=01

- Any vector v=xy can be written as a linear combination of the original basis vectors: v=xe1+ye2. Assuming rotation is a linear transformation, the rotated vector is:

v'=xe1'+ye2'

Substitute your expressions for e1’ and e2’ from part a into  this equation. Write out the resulting expression for x’ and y’ in terms of x, y and θ.

v'=xe1cosθ+e2sin⁡(θ)+y-e1sinθ+e2cosθ

x'=xcosθ+ysinθ

y'=-xsinθ+ycosθ

- Observe that your result can be written as:

Fill in the entries of the 2x2 matrix using trigonometric functions of theta. This is the rotation matrix.

x'y'=cosθsin⁡(θ)-sinθcosθxy